March 23, 1989

ABOUT THESE FILES

Eprommer.doc - this file.

Eprommer.pic - the schematic, a dpaint picture.

Personality.pic - schematics for the personality modules, a dpaint picture.

Eprommer.bas - The basic program, an ascii file.

Eprommer.run - The compiled program.

Exec.bmap - one of those files you have to have to use kickstart routines

from basic.

There are also .info files and maybe a batch file for renaming files with

big names.

WHAT ARE WE HERE FOR?

I designed an EPROM programmer and wrote some software for it. Here

we have the schematics and the source and compiled versions of the

program. I designed this programmer to read and write sound chips for

drum machines and drum pads, so the software has some extra features

that can do some conversion of different data types, as well as raw

AMIGA sound files. Since I've built it, it's been used for other EPROMs

as well. It will read and program 2716 to 27512 EPROMs. It will also

read 24 pin 8k roms as well. Also, it wouldn't take much to modify it

for 32 pin EPROMs up to 4 megabit. The software works pretty well, but

even compiled, it is still written in basic and can't use all the neat

features that C programmers have at their disposal. I promise to learn C

someday. The hardware is first rate. By the way, this whole shebang is

my property and you are only allowed to build it and use the software

personally, no selling or profit-making without my written consent. You

may send the entire arc-ed file to other networks as long as you do not

remove anything from it. I also intend this programmer and the software

for casual use around the house, and can not be responsible for damages

from it, it's use, etc. The EPROMs it makes work fine for me, but don't

put one in a heart-lung machine or an MX missile. Especially not in an

MX missile.(though I have heard that worse parts have gotten into a

few).

WHAT YOU SHOULD DO FIRST

If you haven't executed the execute.me file, you should do that.

Then you can delete it. Print out the pictures. When you look at them,

the aspect ratio may seem a little squat, but they will print nicely.

The pics are 640x408, 2 color. Set preferences pitch to elite.

(important!) In the dpaint print menu, set margins at 1 and 64, page

height 34 lines, %high=100, %wide=100. Use the old preferences printer

drivers, they glitch less from dpaint. There's one on the boards called

newepson.dri that doublestrikes and looks real good. Print out the .bas

version of the program. It's rather large, but hopefully not so

fragmented and batshit that you won't be able to figure it out with a

little help and understanding of the circuit. Actually, the software is

self-explanatory, you can use it without

knowing how it's done.

ABOUT THE CIRCUIT

The first part is the power supply. It requires an ac adapter with

a 12vac rating, meaning more than 12v unloaded. Half wave rectification

is used, and a voltage doubler for the programming voltage. There's a

negative 1.2 volt bias supply so the 317 regulator can swing to 0 volts.

The 317 has three transistors attached to it so it can supply three

programming voltages plus either high or low TTL level, depending on

jumper 8-9 on the personality module. The programming voltage is

software selectable. The first 74HCT374 turns on and sets the

programming voltage. It also can set up one of the other 374's for

latching an address. It does it like this: When we make SEL high, the

first 374 latches onto a byte from the parallel port. If the first

bit(loaddr) is low, we have set up the third 374 so that when we then

make SEL low, the third 374 is clocked, latching the lower address. This

all assumes we have been stuffing the appropriate data out the parallel

port. So we go like this: stuff a command out parallel port, clock SEL

high, stuff a byte of the address out parallel port, clock SEL low,

stuff a command out parallel port, clock SEL high, stuff other byte of

address out parallel port, clock SEL low. Now you have the address set

for the EPROM, and you can read or write it. BUSY is hooked to both OE

on the EPROM and DIR on the 74HCT245, so if we want to read the EPROM we

first should make the parallel port tri-state, then we can pull BUSY low

and the EPROM's data will be sent to the port where we can read it. If

we want to write to the EPROM, we would leave BUSY high, set our

programming voltage, and pulse POUT low. POUT is connected to PGM on the

EPROM, so that's all it takes. On 2732 and 27512 EPROMs, some pins do

more than one thing, so we have to change the program slightly for them,

but basically that's it. The personality modules adjust the pinout for

the various EPROM types. By the way, you will notice there are 3 unused

pins on the command 374. You can hook anything you want there, but I

intended to use them for the extra address lines required by some of the

new jumbo EPROMs in 32 pin packages. Keep your eyes peeled for updates.

HOW TO BUILD IT

I wire-wrapped the thing and put it in a plastic box. I suggest

using a double row header or some kind of connector right on the

perfboard so you don't have a big cable hanging off the end while you

are trying to build it. I also used zero insertion force sockets for the

EPROM and the personality modules. I plugged and glued the ZIF sockets

into wire wrap sockets on the board to get them in the air. I glued all

the sockets to the board. For ICs, you will need three 74HCT374, one

74HCT245, and a 74HCT02. You can probably use LS parts, but HC will

probably not work. You will need a 7805 and a 317 regulator, and if you

use TO-220 packages you won't need to heatsink them. I used a 12 vac,

800ma AC adapter, you can probably use one rated at 500ma. The current

requirements are not steep, but we only half wave rectify plus the

voltage doubler, etc. So much for sentence fragments. Don't substitute

1N4000 types for the 1N914s, we need speed in one place and voltage

drops in another. You must include the .01-5.1k termination on the three

control lines to avoid false clocking when we change data on the

parallel port. I smoked a few EPROMs before I figured that out. I had a

real long cable, and the programming voltage would come on while I was

reading EPROMs. 27512s die when they get 25 volts. You will need some 16

pin dip-type headers to make into personality modules. They are just

jumpers, and most do a couple of EPROM types.

ABOUT THE SOFTWARE

Pretty much the first thing that happens is the allocation of 64k

of ram. This is a scratchpad area where we can move data in and out, cut

and paste. It is the place where all data tranfers must pass. I used the

system's mem-alloc function to allocate this ram, for two reasons. 1)

Basic arrays are limited to 32767 words, 2 bytes shy of 64k. 2) Basic

moves arrays whenever a new variable is defined, and I feel

claustrophobic if I can't create a new variable when I want to. This

causes at least one problem. You must have Exec.bmap in the same

directory as the program. No biggie. Also, this program will not run on

a 512k Amiga. This may be because I do not fully understand the

mem-alloc function. It is not clear to me if this is because I requested

fastram or because this program is huge. It isn't that big, so I assume

there must be a way of requsting fastram if there is fastram, and chip

ram otherwise. This is not a problem except when I need to use a 512k

AMIGA, something I avoid whenever possible. The program then does the

normal menu action, and you may notice the dorky basic-style file

system. Another reason to learn C, then I could use the slick

menu-driven file handlers. It works if you remember filenames. The meat

of the eprom part of the program is something reminiscent of the C64.

That is, where do I poke for the CIA?  That is right, I poke the

hardware directly. So don't try to print from your word processor while

programming EPROMs. Maybe I will OPEN the parallel port officially in a

later version of this program so we won't have any conflicts. Uh-oh,

another small problem. Timing. I added do-nothing loops to get the

one-millisecond programming pulses right. THE TIMING WILL ONLY BE RIGHT

ON THE COMPILED VERSION OF THE PROGRAM. If you insist on programming

from the basic interpreter, you should take out the do-nothing parts or

else zero them out. Also if you do other things while programming

EPROMS, you are likely to get some longer than normal pulses. This is

not likely to cause any problems, but if you are programming 27512s, you

might be tempted to play workbench lander while you wait, and it will

take even longer to program the chip. It is unlikely that the chip will

be harmed. By the way, a 2764 takes about a minute to program, and a

27512 takes about 12 minutes.

The extra features: Two's complement. When going between AMIGA

sounds and, say, E-MU E-DRUM sounds, you must translate 0 to 255 numbers

to -128 to 127 numbers, or vice-versa. That is what Two's complement

will do, take the ram you specify and massage it. By the way, the AMIGA

uses -128 to 127for it's sound files. Expand and compress are not

implemented as of this writing, but they are for translating 12 bits

compressed to 8 bits (such as Linndrum or Drumtraks) into AMIGA samples

or vice-versa. I also intend to include a SHUFFLE routine into this

program, to take alternate address bytes like a 16bit machine into

contiguous bytes like an 8bit machine, useful for data files, not sound

samples.

By the way, use the pulldown menu to exit the program rather than

the close window dot. The system may not properly close all files and

reallocate memory otherwise. Also, to read 2364A ROMs, tell the program

you are reading a 27128 EPROM, but use the 2364A personality module.

Also you must add 4096 to all EPROM addresses, ie to read an entire

2364A, tell the program to start at EPROM address 4096 and to read 8192

bytes. I did it this way because there weren't enough pins on the

personality socket to include A12.

CLOSING COMMENTS

This programmer works pretty well, but if you'd like to improve it

or the software, please feel free. I would love to hear from anyone with

comments, as well. I have yet to find anyone to share hardware hacks on

the AMIGA with. There are a few other projects I will probably upload

eventually, so keep your eyes peeled. By the way, if you haven't built

electronic projects from scratch before, check with an expert before

attempting to build this circuit. The schematic makes certain

assumptions about your electronic prowess.

If you would like to reach me, you may send E-mail to PeopleLink,

my handle is BOFFO*BOB. You can also send  E-mail to MCAB-BBS(a local

Amiga BBS) at 707-964-7114 to BOB BLICK (my real name). My address for

real mail is:

Bob Blick

Box 916

Mendocino, CA 95460